Vivado Boot Camp Day 1 Notes

Resources:
	User Guides

Product Guides

Vivado Synthesis Guide 901 – coding attributes!!!

https://slideplayer.com/slide/5967461/

wp380_Stacked_Silicon_Interconnect_Technology.pdf

https://forums.xilinx.com/t5/Versal-and-UltraScale/Ultrascale-clock-architecture-issue/td-p/782002

ug572-ultrascale-clocking.pdf

http://fpgasite.blogspot.com/2017/05/fpga-internal-tri-state-buses.html

Training Overview:

CLBs:

Slices

LUTs

CIBs (Configurable Interconnect Blocks; Interconnect Fabric).

Memory Resources (UltraRAM vs HBM?):

LUTs

Distributed RAM

Block RAM,

UltraRAM (288Kb SRAM) ???

High Bandwidth memory (HBM) ???

FIFO

DSP Resources

Interfacing:

I/O

SERDES design,

DDR4 Physical Layer Interfaces.

Clocking Resources:

MMCM (clock manager)

PLL

Day 1 Overview:

Architecture.

CLB Resources.
HDL Coding Techniques.

Memory Resources.

DSP Resources.

Day 2 Overview:

IO Resources

Component Mode

Native Mode

Clocking Resources

Day 3 Overview:

Clocking Resources.

Dedicated Hardware (ADCs and such)

Transceivers.

Migration methodology.

Zynq SoC Architecture Overview.

Starting with the 7-series and up the Fourth Generation ASMBL Architecture (everything is organized in columns) is used:

Logic (CLB) Columns.

DSP Columns.

BRAM Columns and UltraRAM Columns.

Clock Buffers and Routing Columns.

IO and CMT (Clock Management Tiles) Columns.

Serial Transceiver Columns.

PCI Express.

HSSIO

Stacked Silicon Interconnect Technology – Stacks multiple FPGAs onto a single chip

Each FPGA Die subunit is referred to as Super Logic Region (SLR)

The chip comprised of SLRs is referred to as Silicon Interposer (Si Interposer).

Adjacent SRLs are interconnected via interposers using Through-Silicn Vias (TSVs).

7-Series SLR layout:

IO columns.

Clock Management Tile (CMT) columns:

MMCM (clock manager).

PLL.

Clock Routing resources.

HSSIO columns (High Speed Source Synchronous Interfaces).

CLB, BRAM, DSP columns.

Architectural Alignment:

Enabled flexible device migration across all 7 series device families by reusing the same components among devices with different layouts.

Clock Regions and IO Banks:

Two types of clocking:

Regional clocking.

Global Clocking.

The boundaries between the Regional and Global clocking is grey area which often overlap.

Clock region aligns with an IO Bank

A clock region is about half the chip.

Within a given clock region, you have 50 CLBs and 50 IOBs.

Typical external clock sourcing involves:
		Synchronous: oscillator -> mmcm -> multiple clocks with a common clock root.

Asynchronous: don’t share the same clock source (handled via fifo, or synchronization circuit).

CLB (Configurable Logic Block) Structure:

CLB contains:

Carry logic.

6-input LUTs.

Registers on the output.

Each CLB contains 2 side by side slices:

A slice contains 4 6-input LUTs.

A slice contains 2 flip-flops per each LUT (8 flip-flops per Slice).

Slice_M (25% of Total Slices; every 4th Slice) – memory capable Slice.

LUTs are user definable as 64x1 RAM/ROM (Distributed RAM), or 32-bit shift register.

Slice_L (75% of Total Slices) – Logic and Carry Slice.

Logic functionality only and not to be used as RAM/ROM.

Has carry logic built-in.

Clocking:

Utilizes

CMT (Clock Management Tile; upto 24 CMTs per device) which contains:

MMCM (1 per CMT)

PLL (1 per CMT)

Clock Buffers to support high clock fan out.

Set up for low-skew clock distribution.

Dedicated Hardware:

Transceivers – support various protocols.

PCIe – various generations and lane widths depending on the FPGA family.

XADC - can be used for internal and external purposes.

Memory:

Distributed RAM - derived from LUTs of Slices.

Block RAM:

Large fully functional RAM Cells

Organized into Columns.

Can be size configured as simple dual-port RAM or true dual-port RAM.

Includes hardened built-in FIFO logic implementation.

Includes hardened Built-in ECC for 64-bit error correction coding per each 36Kb block.

Single bit detection + correction

Multi bit detection.

Can be size configured as a Single 36Kb RAM, or Two 18Kb where one is RAM and the other one is either RAM or FIFO.

Each adjacent 36Kb block contains a dedicated cascade logic to

IO Blocks:

Two types:

HR (High Range) IO: supports voltages up to 3.3V

HP (High Performance) IO: higher performance with more IO delay capability; supports up to 1.8V

Contain:

Simple Input/Output Buffers.

Registers.

Delay Elements.

PLL.

Phase Shifter.

Variety of Voltage Standard support but with this limitations - bank dependent thus one REF and VCCO pin per a bank.

Transceivers:

GTP:

Up to 6.6 Gbps.

Ultra high volume transceivers.

Wire bond package capable.

GTX:

Up to 12.5 Gbps

Support for most common 10 Gbps Protocols

GTH:

Up to 13.1 Gbps

Support 10 Gbps protocols with high FEC overhead.

GTZ:

Up to 28.05 Gbps

Enables 100-400 Gbps communication.

FPGA Family Comparison:

Spartan:

Lowest price and highest performance-per-watt (power efficiency)

Industrial, automotive, infotainment, motor and motion control

Artix:

Lower prices and high performance

Battery powered devices, automotive, commercial digital cameras.

Kintex:

Best price/performance ratio.

Wireless and wired communications, medica, broadcast.

Virtex:

Highest performance and largest capacity.

High-end wired communication,	Test and measurement, Advanced RADAR, High-performance computing.

UltraScale Architecture addresses Interconnect Bottleneck:

Routing delay: dominates overall delay.

Clock Skew: consumes more timing margin

Sub-optimal CLB packing: reduces performance and utilization.

UltraScale Clock Routing architecture:

Previous generations would have clock routed starting in the middle at the single clock root centered in the middle of the chip and fanned out to the rest of the clock region:

UltraScale implements distributed clock network where each clock region has its own clock root in the middle:

Root Clock can be “instantiated” within each clock region (instead of having a single clock root at the center of the device).

Minimized clock route wire length.

Lowers dynamic power due to global clock net switching.

Balanced clock skew across clock distributions.

100s of global clock buffers with 1000s of placement options.

Flexible global clock placement.

UltraScale Block RAM:

Built-in, high-speed memory cascading:

Eliminates CLB usage.

Reduces routing congestion.

Reduces dynamic power consumption.

Enhanced FIFO:

Lower power and greater performance than soft FIFO.

Asymetric read and write port widths for clock domain crossings.

User-accessible power gating of active BRAM:

Reduces dynamic power when access to block RAM contents is temporarily not needed.

UltraScale Transceivers (Faster performance with lower power consumption):

GTH:

Up to 16 Gbps.

Enables:

PCIe Gen4 (16G)

JESD204B (12.5G)

CPRI (16.3G)

Serial Memory (HMC & MoSys)

GTY:

Up to 32 Gbps.

Enables:

28 Gbps backplane support for Nx100G to 400G Systems.

Interlaken.

OTU4 over CFP4.

802.3bj (28G Ethernet Packplane).

Major Power Reduction – 40% lower power for 10G backplanes.

UltraScale built-in Memory Controllers:

DDR4 support (up to 2400 Mbps)

Up to 2 controllers per I/O bank

Integrated PHY – high performance and lower power.

TX Pre-emphasis and RX linera equalization (CTLE)

UltraScale Enhanced PCIe Gen3 Integrated Core:

Built on existing Virtex-7 XT/HT core.

Gen3 support in all devices and speed grades.

Transaction Layer Bypass mode allows additional physical and virtual functions for diverse topologies.

Integrated circuitry for 100 ms configuration for PCIe specifications compliance.

CLBs:

CLB Resources (7 Series Architecture) Coved:

Fabric Routing and CLB Arrangement.

Available CLB and Slice resources in 7 Series FPGAs.

Distributed RAM and SRL (Shift Register LTU) Capability.

CLBs are utilized as primary resources for

Combinatorial functions.

Flip-Flops.

CLBs contain 2 types of slices:
		SLICE_L

Logic and Carry Slice.

Carry chain runs vertically thru the Logic Column from one slice to the next slice.

SCLICE_M – Memory capable slice (Distributed Memory)

Floor Planning layout:

Notice from the figure above how each CLB can be referenced by the Y coordinate of the Slice and each Slice within the CLB can be referred to by both the CLB’s Y coordinate and the Slice’s X coordinate.

Slice details:

4 6-input LUTs

Wide multiplexers

Carry chain

4 flops/lateches per slice

4 additional output flops per slice

The 6-input LUTS details:

Implemented as 2 mux’d 5-input LUTs with shared inputs

One or two outputs

Can be used as either:

a single function with up to 6 variables

two independent functions with up to 5 variables.

Wide Multiplexers: