Vivado Boot Camp Day 1 Notes

Resources:
	User Guides

Product Guides

Vivado Synthesis Guide 901 – coding attributes!!!

https://slideplayer.com/slide/5967461/

Training Overview:

CLBs:

Slices

LUTs

CIBs (Configurable Interconnect Blocks; Interconnect Fabric).

Memory Resources (UltraRAM vs HBM?):

LUTs

Distributed RAM

Block RAM,

UltraRAM (288Kb SRAM) ???

High Bandwidth memory (HBM) ???

FIFO

DSP Resources

Interfacing:

I/O

SERDES design,

DDR4 Physical Layer Interfaces.

Clocking Resources:

MMCM (clock manager)

PLL

Day 1 Overview:

Architecture.

CLB Resources.
HDL Coding Techniques.

Memory Resources.

DSP Resources.

Day 2 Overview:

IO Resources

Component Mode

Native Mode

Clocking Resources

Day 3 Overview:

Clocking Resources.

Dedicated Hardware (ADCs and such)

Transceivers.

Migration methodology.

Zynq SoC Architecture Overview.

Starting with the 7-series and up the Fourth Generation ASMBL Architecture (everything is organized in columns) is used:

Logic (CLB) Columns.

DSP Columns.

BRAM Columns and UltraRAM Columns.

Clock Buffers and Routing Columns.

IO and CMT (Clock Management Tiles) Columns.

Serial Transceiver Columns.

PCI Express.

HSSIO

Stacked Silicon Interconnect Technology – Stacks multiple FPGAs onto a single chip

Each FPGA Die subunit is referred to as Super Logic Region (SLR)

The chip comprised of SLRs is referred to as Silicon Interposer (Si Interposer).

Adjacent SRLs are interconnected via interposers using Through-Silicn Vias (TSVs).

7-Series SLR layout:

IO columns.

Clock Management Tile (CMT) columns:

MMCM (clock manager).

PLL.

Clock Routing resources.

HSSIO columns (High Speed Source Synchronous Interfaces).

CLB, BRAM, DSP columns.

Architectural Alignment:

Enabled flexible device migration across all 7 series device families by reusing the same components among devices with different layouts.

Clock Regions and IO Banks:

Two types of clocking:

Regional clocking.

Global Clocking.

The boundaries between the Regional and Global clocking is grey area which often overlap.

Clock region aligns with an IO Bank

A clock region is about half the chip.

Within a given clock region, you have 50 CLBs and 50 IOBs.

Typical external clock sourcing involves:
		Synchronous: oscillator -> mmcm -> multiple clocks with a common clock root.

Asynchronous: don’t share the same clock source (handled via fifo, or synchronization circuit).

CLB (Configurable Logic Block) Structure:

CLB contains:

Carry logic.

6-input LUTs.

Registers on the output.

Each CLB contains 2 side by side slices:

A slice contains 4 6-input LUTs.

A slice contains 2 flip-flops per each LUT (8 flip-flops per Slice).

Slice_M (25% of Total Slices; every 4th Slice) – memory capable Slice.

LUTs are user definable as 64x1 RAM/ROM (Distributed RAM), or 32-bit shift register.

Slice_L (75% of Total Slices) – Logic and Carry Slice.

Logic functionality only and not to be used as RAM/ROM.

Has carry logic built-in.

Clocking:

Utilizes

CMT (Clock Management Tile; upto 24 CMTs per device) which contains:

MMCM (1 per CMT)

PLL (1 per CMT)

Clock Buffers to support high clock fan out.

Set up for low-skew clock distribution.

Dedicated Hardware:

Transceivers – support various protocols.

PCIe – various generations and lane widths depending on the FPGA family.

XADC - can be used for internal and external purposes.

Memory:

Distributed RAM - derived from LUTs of Slices.

Block RAM:

Large fully functional RAM Cells

Organized into Columns.

Can be size configured as simple dual-port RAM or true dual-port RAM.

Includes hardened built-in FIFO logic implementation.

Includes hardened Built-in ECC for 64-bit error correction coding per each 36Kb block.

Single bit detection + correction

Multi bit detection.

Can be size configured as a Single 36Kb RAM, or Two 18Kb where one is RAM and the other one is either RAM or FIFO.

Each adjacent 36Kb block contains a dedicated cascade logic to

IO Blocks:

Two types:

HR (High Range) IO: supports voltages up to 3.3V

HP (High Performance) IO: higher performance with more IO delay capability; supports up to 1.8V

Contain:

Simple Input/Output Buffers.

Registers.

Delay Elements.

PLL.

Phase Shifter.

Variety of Voltage Standard support but with this limitations - bank dependent thus one REF and VCCO pin per a bank.

Transceivers:

GTP:

Up to 6.6 Gbps

Ultra high volume transceivers.

Wire bond package capable.

GTX:

Up to 12.5 Gbps

Support for most common 10 Gbps Protocols

GTH:

Up to 13.1 Gbps

Support 10 Gbps protocols with high FEC overhead.

GTZ:

Up to 28.05 Gbps

Enables 100-400 Gbps communication.

FPGA Family Comparison:

Spartan:

Lowest price and highest performance-per-watt (power efficiency)

Industrial, automotive, infotainment, motor and motion control

Artix:

Lower prices and high performance

Battery powered devices, automotive, commercial digital cameras.

Kintex:

Best price/performance ratio.

Wireless and wired communications, medica, broadcast.

Virtex:

Highest performance and largest capacity.

High-end wired communication,	Test and measurement, Advanced RADAR, High-performance computing.

UltraScale Architecture addresses Interconnect Bottleneck:

Routing delay: dominates overall delay.

Clock Skew: consumes more timing margin

Sub-optimal CLB packing: reduces performance and utilization.

UltraScale Clock Routing architecture: